# What to Do When Your Garbage Disposal Stops Working

A garbage disposal that stops working doesn’t always mean it needs to be replaced. Most of the time, it’s a simple issue like a jam, a reset, or a power problem.

Here’s how to work through it step by step.

## Step 1: Pay Attention to What It’s Doing

Turn the switch on briefly after confirming it’s safe:

- If you hear a humming sound, the motor is likely jammed.

- If there’s no sound at all, it’s likely an electrical issue.

- If it clicks and shuts off, the overload protection has tripped.

This helps narrow down what you’re dealing with.

## Step 2: Turn Everything Off

Before you do anything:

- Turn off the disposal switch.

- Unplug it under the sink if it has a plug.

- If it’s hardwired, switch off the breaker.

Make sure there is no power going to the unit before checking anything inside or underneath it.

## Step 3: Press the Reset Button

Most garbage disposals have a small reset button on the bottom.

- Press the button firmly.

- Wait about 30 to 60 seconds.

- Turn it back on and test it again.

If it was overloaded, this will usually get it working again.

## Step 4: Check for a Jam

If you hear a hum but the blades aren’t spinning:

- Insert an Allen wrench into the bottom slot.

- Turn it back and forth slowly to free the impeller.

- Keep turning until it moves more freely.

Do not force anything with tools that don’t fit properly.

## Step 5: Remove Any Blockages

If it’s still not working:

- Use tongs or pliers to remove any visible debris.

- Common causes include bones, utensils, or fibrous food like celery or potato peels.

Once cleared, run cold water and test it again.

## Step 6: Check the Power Source

If it still won’t run:

- Check your breaker panel for a tripped breaker.

- Reset any GFCI outlets nearby.

- Make sure the unit is securely plugged in.

Sometimes the issue is just a tripped breaker or outlet, not the disposal itself.

## When to Call a Plumber

You should bring in a professional if:

- The disposal still won’t turn on after resetting.

- The breaker keeps tripping.

- You notice a burning smell.

- The unit is leaking from the bottom.

- It keeps giving you problems and is older.

At that point, it may be more cost-effective to repair or replace it.

If you need assistance fixing your garbage disposal or don’t feel comfortable working on it, call our licensed and insured technicians. Our College Station team is available 27/4 at 979-260-2020.